### 1. download_graph.py (The Skeleton)

What it does: It uses the osmnx library to download the entire drivable road network of Bangalore and saves it as bangalore_graph.graphml.

Why it matters:

- It turns the city into a mathematical graph. Intersections become nodes, and the roads between them become edges (roughly 393,000 of them).

- This is the backbone of the entire project. Every single RF signal calculation, OOKLA speed test, and computer vision canopy check will eventually be glued to one of these specific edges.

### 2. download_building.py (The Obstacles)

What it does: It sends a highly specific bounding-box query to the OpenStreetMap Overpass API ([out:json][timeout:300]; way["building"]...). It pulls down the polygon shapes of every building in Bangalore.

Why it matters:

- Cellular signals (especially 5G) bounce off or get absorbed by concrete.

- The script actively looks for a height or building:levels tag. If a building doesn't have a height listed, it defaults to 10 meters (about 3 stories).

- It saves this out as bangalore_buildings.geojson. Later on, the system will shoot digital "rays" at these polygons to figure out how much of the sky is blocked from any given road.

### 3. filter_towers.py (The Transmitters)

What it does: This script takes massive CSV dumps of global cell tower locations (from OpenCelliD) and shrinks them down into usable, carrier-specific files for Bangalore.

- Geographic Filter: It immediately discards any tower outside the Bangalore bounding box.

- Carrier Filtering (The MNC Map): It uses Mobile Network Codes (MNC) to identify who owns the tower. For example, MNC 50 is Jio, 45 is Airtel, 20 is Vi, and 01 or 34 is BSNL. It splits the towers into four separate datasets (Parquet files) based on these codes.

- Frequency Inference: This is the most important technical step. The raw data tells us the radio type (GSM, LTE, NR/5G), but the physics equations later on need the exact frequency in Megahertz (MHz). The script maps radio types to default frequencies based on Indian telecom standards (e.g., Airtel NR = 3500 MHz, Jio LTE = 2300 MHz).

Why it matters: Frequency is the bedrock of radio physics. A 3500 MHz 5G signal carries a massive amount of data, but it is incredibly fragile—a thick tree canopy or a concrete building will block it easily. On the other hand, an 850 MHz BSNL signal carries less data but can penetrate walls and travel much further. By tagging every tower with its specific frequency, the pipeline ensures the downstream signal simulation behaves like the real physical world.

4. compute_features.py

### 1. Spatial Indexing (The Setup)

If the code checked every road segment against every building one by one, it would take weeks to run ($O(N^2)$ complexity). Instead, step one is building spatial indexes.

- STRtree for Buildings & Labels: It loads the polygons from bangalore_buildings.geojson and the OOKLA speedtest tiles into an STRtree (Sort-Tile-Recursive tree via Shapely). This allows the script to instantly query "which buildings are within 200 meters of this exact coordinate?"

- KDTree for Towers: It takes the coordinates of the Jio, Airtel, Vi, and BSNL towers and puts them into SciPy KDTrees. This makes finding the nearest cell towers to any point on the map an $O(\log N)$ operation.

### 2. Ray-Casting the Sky View Factor (compute_svf)

This function behaves exactly like a 2D Lidar scanner attached to a rover, but applied to a digital map.

- It takes the midpoint of a road segment.

- It casts 36 rays outward (one every 10 degrees), each extending 200 meters.

- It uses the STRtree to instantly grab nearby buildings, and then checks if the ray's LineString intersects the building's Polygon.

- If it hits a building, that ray is "blocked".

- The final SVF is a float between 0.0 and 1.0 (e.g., 27 unblocked rays / 36 total = 0.75 SVF). This is later inverted to calculate building_density.

### 3. The RF Physics Engine (compute_rf_score & hata_path_loss_batch)

This is the core signal simulation. For a given road segment and a given carrier (e.g., Airtel):

- Find Towers: It queries the KDTree for all Airtel towers within a 5km radius. If none exist, it immediately returns a baseline dead-zone score of 20.0.

- Path Loss Math: It calculates the distance to each tower using the Haversine formula, then pushes the tower frequencies and distances through the Okumura-Hata urban path loss model.

- Power-Sum Combining: It assumes standard transmission power (46 dBm) and antenna gain (15 dB). It calculates the resulting RSRP (Reference Signal Received Power) for each tower, applies a multiplier based on the technology (5G NR = 1.0, GSM = 0.1), and combines them in the linear domain to simulate how signals overlap.

- The Obstruction Penalty: It takes the combined signal and subtracts up to 15 dB based on the SVF calculated earlier. A dense concrete canopy kills the signal.

- Normalization: The final dBm value is normalized to a 0-100 score.

### 4. 3D Terrain Shadowing (compute_terrain_shadow)

Buildings aren't the only obstacles; the earth itself gets in the way.

- This function finds the single absolute closest tower to the road segment.

- It draws a line between them and samples 10 points along that line.

- For each point, it queries the NASA SRTM raster file (sample_elevation) to get the physical ground height.

- If the physical ground at any of those 10 points is taller than the expected line-of-sight path from the top of the tower to the road, it throws a terrain_shadow = 1 flag. Later in the script, this flag slashes the RF scores by 60%.

Here is exactly how Teammate 1 designed the integration logic, priority weights, and penalties in the physics engine.

### 1. The Tower Reliability Weight

The OpenCelliD dataset is crowdsourced, which means some tower locations are guessed by one phone, while others are verified by thousands.

- The Weight: The code extracts the samples column from the tower data (which counts how many devices saw that tower).

- The Integration: When calculating the raw signal power, it multiplies the signal by min(samples / 100, 1.0). If a tower only has 10 samples, its impact is slashed to 10%. If it has 100+ samples, it gets full weight. This prevents "ghost towers" with bad coordinates from ruining the simulation.

### 2. The Technology Hierarchy (Bandwidth > Raw Signal)

Not all radio waves are created equal. A slightly weak 4G signal is still infinitely better for routing than a booming 2G signal.

- The Weight: The script applies a hardcoded TECH_MULT dictionary to the physical signal strength:

  - 5G NR: 1.0 (100% weight)

  - 4G LTE: 0.75 (75% weight)

  - 3G HSPA: 0.4 (40% weight)

  - 2G GSM: 0.1 (10% weight)

- The Integration: Before adding up the signals, they are scaled by these multipliers. The system fundamentally prioritizes modern, high-capacity bands.

### 3. Power-Sum Combining (No "Winner Takes All")

If you are standing on a street corner and your phone can see three Airtel towers, the script doesn't just pick the closest one and ignore the rest.

- The Integration: It calculates the RSRP (in decibels) for every tower within 5km. It converts those logarithmic decibels into linear power watts, adds them all together (multiplying by the technology and sample weights mentioned above), and then converts the total back into decibels. This accurately simulates how overlapping cell sectors reinforce each other.

### 4. The Deductions: Buildings vs. Earth

Once the raw, combined signal from the towers hits the road segment, the obstacle datasets take their cuts. They do this in very different ways:

- The Building Penalty (Scaled Deduction): Concrete absorbs signal, but radio waves can bounce down "urban canyons."

  - Integration: The script uses the Sky View Factor (SVF). It subtracts up to 15 dBm from the signal based on the formula 15 * (1 - svf). If the sky is 100% blocked, you lose 15 dBm. If it's 50% blocked, you lose 7.5 dBm. It's a sliding scale.

- The Terrain Penalty (The Absolute Veto): Unlike buildings, a massive hill made of dirt and rock will completely sever a line-of-sight signal.

  - Integration: If the NASA SRTM dataset shows that the ground elevation interrupts the line-of-sight between the road and the nearest tower, the script applies a brutal 0.4 multiplier to the entire final score. It immediately chops 60% of the signal score away. Earth overrules everything.

### 5. Ground Truth Priority (The Target)

At the very end of compute_features.py, the script tries to attach a real-world label so the neural network has something to learn from.

- Primary Truth (OOKLA): The script queries the OOKLA dataset. If the road segment falls inside an OOKLA H3 hex tile, it pulls the avg_d_kbps (average download speed) and sets that as the absolute truth for that segment.

- Secondary Truth (CellMapper): According to the pipeline doc, if OOKLA data is missing for that specific spot, it falls back to looking for CellMapper drive-test points within 50 meters. OOKLA wins because speed tests represent actual user experience, whereas CellMapper is just raw radio strength.

5. gnn_pipeline.py

### The "Why": The Missing Label Problem

Phase 1 and Phase 2 gave us physical calculations (like Sky View Factor and RF path loss). But to know if those calculations actually translate to real-world internet speed, we need ground truth data (OOKLA speed tests).

The problem? OOKLA data only covers about 40–60% of Bangalore's roads. If a user tries to route through a residential street with no OOKLA data, a standard machine learning model (like Random Forest or XGBoost) would just guess blindly based on the physical features.

Graph Neural Networks (GNNs) solve this. They understand that roads are physically connected. If Road A has a booming 5G signal, and Road B intersects it and shares the same open sky, Road B probably has a great signal too. The GNN "flows" the ground-truth labels from the known roads into the unknown roads through the graph structure.

### Code-by-Code Breakdown

#### 1. Label Normalization (normalise_labels)

You can't just feed raw Megabits per second (Mbps) into a neural network because the variance is insane (some spots have 2 Mbps, some have 800 Mbps).

- The Math: It takes the raw OOKLA kbps and applies a log-scale percentile stretch: $\log_{10}(\text{kbps} + 1)$.

- Why it matters: It chops off the extreme outliers (the top 2% and bottom 2%) and squashes the real-world speeds into a clean, smooth 0 to 100 score.

#### 2. Building the PyTorch Geometric Graph (build_pyg_graph)

This function translates the city map into a format PyTorch can train on.

- The Nodes (X): It takes the 13 features calculated in Phase 2 (SVF, elevation, segment length, RF scores) and normalizes them (StandardScaler) so the neural network doesn't get confused by different units (like comparing meters of elevation to decibels of signal).

- The Edges (edge_index): It creates links between road segments. If two segments belong to the same OSM way (i.e., they flow into each other), they are mathematically connected in the graph.

#### 3. The Masterstroke: 20x20 Spatial Grid Split

Look at the Spatial Grid Split section. This is the mark of a very experienced ML engineer.

- The Trap: Usually, ML engineers randomly split 80% of data for training and 20% for testing. In spatial data, if you do this, the model just memorizes the training road right next to the testing road. This is called data leakage.

- The Solution: Teammate 2 divides Bangalore into a 20x20 grid (400 blocks). They assign entire blocks to Train, Validate, or Test. The model is forced to learn the actual physics of the signals, because it is tested on entirely different geographic neighborhoods than it was trained on.

#### 4. The Model Architecture (NeuralPathGNN)

This is the neural network itself. It uses GraphSAGE (SAGEConv).

- Two Layers: It only uses two layers. In GNNs, if you use too many layers, every node eventually averages out to look exactly the same (a problem called over-smoothing). Two layers mean a road only looks at its immediate neighbors and its neighbors' neighbors.

- aggr='max' (Max Aggregation): This is a brilliant, telecom-specific choice. Usually, GNNs average ('mean') the features of their neighbors. But imagine an intersection: three roads are in a concrete canyon (dead zones), and one road has a clear line-of-sight to a cell tower. If you average them, the strong signal gets washed out. By using 'max' pooling, the GNN preserves the strong line-of-sight signal spikes.

Show me the visualization

#### 5. Training (train)

- Huber Loss: Instead of standard Mean Squared Error (MSE), it uses Huber Loss (F.huber_loss). Cellular speed tests have crazy outliers (someone standing right next to a 5G mmWave node getting 1,000 Mbps). MSE punishes the model too heavily for missing these outliers. Huber loss is more forgiving to extreme anomalies, creating a more stable routing model.

- Early Stopping: It monitors the Validation set. If the error hasn't improved in 120 epochs (PATIENCE), it stops training to prevent overfitting.

#### 6. Exporting to the Backend (export_geojson)

Once trained, the model predicts a composite_score (0-100) for all 393,000 segments.

- Carrier Specificity: The GNN only predicts an overall composite score. To get Jio, Airtel, Vi, and BSNL specific scores, it looks back at the physical RF estimates from Phase 2. It calculates a ratio (e.g., "Airtel is 20% stronger than the average here") and applies that ratio to the GNN's composite score.

- Confidence Score: It tags segments with a confidence metric. If the segment had actual ground truth data nearby, confidence is high (~0.95). If the GNN had to guess completely based on the graph, confidence is lower (~0.60).

- The Output: It bundles the geometries, scores, traversal times, and dominant bands into scored_segments.geojson.

6.cv_demo.py and cv_visualise.py.

### The "Why": The Blind Spot of Maps

Teammate 1's physics engine calculates Sky View Factor (SVF) by shooting digital rays at 2D OpenStreetMap (OSM) building polygons. The fatal flaw? OSM doesn't map trees, metro pillars, or temporary construction scaffolding. If you drive down 26th Main Road in Jayanagar, the map sees open sky between the buildings. The physical reality, however, is a dense tunnel of tree canopy. That canopy blocks 5G signals just as effectively as concrete.

To prove this to the hackathon judges, Teammate 2 built a pipeline that uses street-level imagery to calculate the true SVF.

### Code-by-Code Breakdown (cv_demo.py)

#### 1. Image Acquisition (fetch_image)

The script uses the Mapillary API. Mapillary is a crowdsourced street-view database.

- It targets a specific road segment (OSM ID 780853199 in Jayanagar).

- It pulls three specific, high-resolution street-level images taken from that exact road segment.

#### 2. Semantic Segmentation (compute_camera_svf)

YOLOv8 draws bounding boxes around objects. This task requires Semantic Segmentation, which categorizes every single pixel in an image.

- The Model: They load SegFormer-B2 from Hugging Face. SegFormer is a transformer-based vision model developed by NVIDIA. It's incredibly efficient, which is perfect for running locally on an RTX 4060.

- The Training Data (ADE20K): The model was finetuned on the ADE20K dataset, which maps 150 different environmental classes to specific numbers.

  - Class 2 is Sky.

  - Class 4 is Tree.

  - Class 1 is Building.

- The Pixel Math: For each image, the model spits out a map of these class numbers. The script simply counts the pixels: (seg_map == 2).float().mean(). This gives the exact percentage of the image that is unobstructed sky.

- The Worst-Case Scenario: It processes all 3 images for the street and takes the minimum sky fraction (min(sky_fractions)). Cellular routing cares about the worst choke point. If one image shows 80% sky and another shows 30% sky (under a thick tree), the signal drops to the 30% level.

#### 3. Recalculating the Physics (hata_rsrp_from_svf)

Now the script has two competing versions of reality:

- Geometric SVF: What the OSM map says (e.g., 0.72 or 72% sky).

- Camera SVF: What SegFormer actually saw (e.g., 0.38 or 38% sky).

It plugs both of these numbers back into the exact same Hata obstruction formula that Teammate 1 built: base_rsrp - 10 * (1 - svf) * building_density * 20.

#### 4. The Proof (The Main Block)

The script pulls the real-world OOKLA speed test reading for that specific road segment and converts it to decibels (-87.2 dBm).

- The Geometric model guesses the signal is strong (-82 dBm) because it doesn't know about the trees. It has an error of ~5 dB.

- The Camera model sees the canopy, drops the SVF, and guesses the signal is weak (-91 dBm). It has an error of only ~3.8 dB.

By applying computer vision, the prediction accuracy drastically improves.

### Visualizing the Data (cv_visualise.py)

This script exists purely to make the backend math look spectacular for the frontend demo.

- It takes the raw output matrix from SegFormer and maps the ADE20K classes to hardcoded RGB colors (Sky = Light Blue, Trees = Forest Green).

- It uses torch.nn.functional.interpolate to upscale the segmentation map back to the original image resolution.

- It blends the segmentation map with the original photo at 60% opacity, generating the images that will sit in the frontend comparison panel.

Show me the visualization

This completes the offline processing! Between the massive physics simulation, the GNN spatial reasoning, and the CV canopy detection, the system has a highly accurate, city-wide map of cellular dead zones.